Министерство образования и науки РФ

Федеральное государственное бюджетное образовательное учреждение

высшего образования «Московский политехнический университет»

факультет информационных технологий

Кафедра СМАРТ-технологий

Дисциплина: Нейронные сети в задачах технического зрения и управления

Отчёт по лабораторной работе №3

«Применение свёрточных нейронных сетей»

Работу выполнил_и

студент_ка 3 курса

очного отделения

<ФИО>

Научный руководитель

_______________________

<ФИО>

Цель работы

Разработать алгоритм классификации стандартизованных изображений с помощью свёрточной нейронной сети.

Задачи

- Подготовить приложение для генерации обучающей выборки из исходного набора изображений;

- Разработать функцию расчета свёрточной карты для выбранного ядра;

- Разработать класс, реализующий функциональность свёрточной сети для классификации изображений;

- Разработать функцию обучения с учителем для нейтронной сети по подготовленной ранее обучающей выборке.

Ход работы

В начале было разработано приложение, генерирующее набор изображений, размером 20x20 пикселей для обучения и проверки работы нейросети из любого цветного или ч/б изображения.

В изображения подмешивается яркостный шум, поворот, сдвиг пикселей, изменение яркости. Имеется возможность настройки каждого параметра. Полученные изображения сохраняются в отдельные файлы.

Основное приложение использует данные изображения для обучения и проверки работы нейросети.

Полученные изображения преобразуются в одномерный массив 0-1 и сопоставляясь с «метками»:

-------------------- INPUT DATA --------------------

Shape of loaded_imagess: (4, 50, 20, 20)

Shape of loaded_labels: (4, 50, 1)

Arrays:

[[[[0.37254902 0.22745098 0.1372549  ... 0.07058824 0.2627451

...

[1.         1.         1.         ... 1.         1.

1.        ]]

...

[[0.03921569 0.07058824 0.04313725 ... 0.55686275 0.61176471

1.        ]

...

[0.61568627 0.55686275 0.49411765 ... 0.03921569 0.60784314

1.        ]]]]

[[[0]

[0]

...

[3]

[3]]]

----------------------------------------------------

Далее выполняется генерация ядер свёртки (5x5) и (2x2) случайным образом. Диапазон значений {0; 1}. Также имеется возможность сохранить или загрузить ядра из файла.

Первые фильтры применяются со страйдом 1, вторые – со страйдом 2. В итоге, на выходе получается изображение 8x8 пикселей. После операции свёртки производится нормализация полученных изображений. Имеется возможность предпросмотра операций свёртки и пуллинга. А также, загрузка и сохранение результатов в файл.

После свёртки, изображение преобразуется в одномерный массив с диапазоном [0; 1] и подаётся на вход MLP-сети, структура которой выглядит следующим образом:

Обучение производиться только оранжевых весов и выглядит следующим образом:

- За первый слой принимается массив входных данных (изображения)

- Далее, вычисляется скалярное произведение первого слоя на первые веса (значения которых {-1; 0; 1}). Результат проходит через функцию активации:

def layer_0_activator(weights_sum):
    result = [[0] * weights_sum[0]] * weights_sum
    for i in range(len(weights_sum)):
        sample_result = [0] * weights_sum[0]
        for k in range(len(weights_sum[i])):

            threshold = 1.8 
            if weights_sum[i][k] >= threshold:
                sample_result[k] = weights_sum[i][k] - 2.2
            else:
                sample_result[k] = 0

        result[i] = sample_result
    result = np.array(result)
    return np.array(result)

- Результат считается вторым слоем. Далее производится скалярное произведение этого [второго] слоя на вторые веса (значения которых [0; 1]). Результат пропускается через сигмоидную функцию активации:

def sigmoid(x):
    return 1 / (1 + np.exp(-x))

- В итоге, последний (третий) слой содержит 4 элемента.

- Вычисляется ошибка (тот выход, который должен соответствовать метке принимается за 1, остальные три – за 0.).

- На основе ошибки вычисляются корректирующие значения для весов

adjustments_l2 = output_l1.T.dot(error_l2 * (output_l2 * (1 - output_l2)))

- Эти корректировки прибавляются к весам второго слоя.

В среднем, необходимо 2 итерации чтобы достичь точности в 80-90% и 30-40 итераций, чтобы достичь точности в 95-100%

-------------------- I: 0 --------------------

accuracy: 0.145

-------------------- I: 1 --------------------

accuracy: 0.85

-------------------- I: 2 --------------------

accuracy: 0.925

-------------------- I: 3 --------------------

accuracy: 0.935

...

-------------------- I: 29 --------------------

accuracy: 0.98

-------------------- I: 30 --------------------

accuracy: 0.98

Обученные веса можно сохранить в файл для дальнейшего использования.

Результат предсказания отображается в виде выходов последнего слоя. А также имеется графический предпросмотр работы MLP в виде слоёв и весов.

Вывод

В ходе данной работы была создана свёрточная нейросеть, в начале выполняющая операции свёртка и пуллинга, а также, дальнейшую классификацию с высокой точностью.

Исходный код генератора изображений

import os
import numpy as np
import cv2
import sys
from PyQt5 import QtWidgets
from PyQt5.QtGui import QPixmap
import qimage2ndarray
import random
from tkinter import filedialog
import tkinter

root = tkinter.Tk()
root.withdraw()

import gui_3_Collector


def valmap(value, istart, istop, ostart, ostop):
    return ostart + (ostop - ostart) * ((value - istart) / (istop - istart))


class CollectorApp(QtWidgets.QMainWindow, gui_3_Collector.Ui_MainWindow):
    def __init__(self):
        super().__init__()
        self.setupUi(self)
        self.btn_open_file.clicked.connect(self.open_file)
        self.btn_candidates_generate.clicked.connect(self.generate_candidates)

        # self.cvl_original.setStyleSheet("background-color: lightgreen")
        # self.cvl_candidates.setStyleSheet("background-color: lightgreen")

        self.original_image = None
        self.candidates_image = None

    def open_file(self):
        file_path = filedialog.askopenfilename()
        temp_image = cv2.cvtColor(cv2.imread(file_path), cv2.COLOR_BGR2GRAY)
        self.original_image = cv2.resize(temp_image, (20, 20), interpolation=cv2.INTER_NEAREST)
        self.cvl_original.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
                cv2.cvtColor(cv2.resize(self.original_image, (80, 80), interpolation=cv2.INTER_NEAREST),
                             cv2.COLOR_GRAY2RGB))))

        print(file_path)

    def generate_candidates(self):
        self.candidates_image = np.zeros(shape=[320, 320], dtype=np.uint8)  # [16 X 16] images or [16*20 X 16*20] pixels
        generated_images = []

        # Generation of modified images
        for n in range(self.spin_cindidates_n.value()):
            temp_image = self.original_image.copy().astype('float32')

            # Brightness noise
            if self.slider_brightness.value():
                temp_image += random.randrange(-self.slider_brightness.value(), self.slider_brightness.value())

            temp_image = np.clip(temp_image, 0, 255)
            temp_image = temp_image.astype('uint8')

            # Pixel noise
            if self.slider_noise.value():
                for i in range(20):
                    for j in range(20):
                        if random.randrange(0, 2) and temp_image[i][j] > 0:
                            brightness_pixel = temp_image[i][j] + random.randrange(-self.slider_noise.value(),
                                                                                   self.slider_noise.value())
                            if brightness_pixel > 255: brightness_pixel = 255
                            if brightness_pixel < 0: brightness_pixel = 0
                            temp_image[i][j] = brightness_pixel

            # Rotation
            if self.slider_rotation.value() > 0:
                num_rows, num_cols = temp_image.shape[:2]
                rotation_matrix = cv2.getRotationMatrix2D(
                    (num_cols / 2, num_rows / 2),
                    random.randrange(-self.slider_rotation.value(), self.slider_rotation.value()), 1)
                temp_image = cv2.warpAffine(temp_image, rotation_matrix, (num_cols, num_rows))

            # Pixel shift
            if self.slider_shift.value() > 0:
                shift_size_x = random.randrange(-self.slider_shift.value(), self.slider_shift.value())
                shift_size_y = random.randrange(-self.slider_shift.value(), self.slider_shift.value())
                rows, cols = temp_image.shape
                M = np.float32([[1, 0, shift_size_x], [0, 1, shift_size_y]])
                temp_image = cv2.warpAffine(temp_image, M, (cols, rows))

                generated_images.append(temp_image)

        if not os.path.exists('LR3_data'):
            os.makedirs('LR3_data')

        image_counter = 0
        for y in range(16):
            for x in range(16):
                temp_image = generated_images[image_counter]

                # Save to file
                cv2.imwrite('LR3_data/' + str(self.spin_cindidates_file.value())
                            + '_' + str(image_counter) + '.bmp', temp_image)

                self.candidates_image[y * 20: y * 20 + 20, x * 20: x * 20 + 20] = temp_image  # y:y + h, x:x + w
                image_counter += 1
                if image_counter == len(generated_images):
                    break
            if image_counter == len(generated_images):
                break

        self.cvl_candidates.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
            cv2.cvtColor(cv2.resize(self.candidates_image, (512, 512),
                                    interpolation=cv2.INTER_NEAREST), cv2.COLOR_GRAY2RGB))))


if __name__ == '__main__':
    app = QtWidgets.QApplication(sys.argv)
    app.setStyle("fusion")
    window = CollectorApp()
    window.show()
    app.exec_()

Исходный код основной программы

import threading

import numpy as np
import cv2
import sys
from PyQt5 import QtWidgets
import random
from PyQt5.QtGui import QPixmap
import qimage2ndarray
import pickle

import gui_3_CNN


def sigmoid(x):
    return 1 / (1 + np.exp(-x))


def dot_0_layer(input_layer, synaptic_weights):
    return layer_0_activator(np.dot(input_layer, synaptic_weights.T))


def valmap(value, istart, istop, ostart, ostop):
    return ostart + (ostop - ostart) * ((value - istart) / (istop - istart))


def layer_0_activator(weights_sum):
    # return max(0, weights_sum)
    result = [[0] * weights_sum[0]] * weights_sum
    for i in range(len(weights_sum)):
        sample_result = [0] * weights_sum[0]
        for k in range(len(weights_sum[i])):

            threshold = 1.8  # 1.79
            if weights_sum[i][k] >= threshold:
                sample_result[k] = weights_sum[i][k] - 2.2  # 0.79
            else:
                sample_result[k] = 0

        result[i] = sample_result
    result = np.array(result)
    # exit(0)
    return np.array(result)


class CNNApp(QtWidgets.QMainWindow, gui_3_CNN.Ui_MainWindow):
    def __init__(self):
        self.DEBUG = True

        super().__init__()
        self.setupUi(self)
        self.btn_load_images.clicked.connect(self.load_images)
        self.btn_filter_1_generate.clicked.connect(self.filter_1_generate)
        self.btn_filter_1_load.clicked.connect(self.filter_1_load)
        self.btn_filter_1_save.clicked.connect(self.filter_1_save)
        self.btn_filter_2_generate.clicked.connect(self.filter_2_generate)
        self.btn_filter_2_load.clicked.connect(self.filter_2_load)
        self.btn_filter_2_save.clicked.connect(self.filter_2_save)
        self.btn_filters_load.clicked.connect(self.filters_load)
        self.btn_filters_save.clicked.connect(self.filters_save)

        self.btn_apply_filters.clicked.connect(self.apply_filters)
        self.btn_preview_filters.clicked.connect(self.preview_filters)

        self.btn_start_training.clicked.connect(self.start_training)
        self.btn_predict.clicked.connect(self.predict_test_image)
        self.btn_save_to_file.clicked.connect(self.save_model_to_file)
        self.btn_load_from_file.clicked.connect(self.load_model_from_file)
        self.test_values = False

        self.synaptic_weights_0 = np.array([])
        self.synaptic_weights_1 = np.array([])

        self.loaded_images = []
        self.loaded_labels = []
        self.cnn_filters_1 = []
        self.cnn_convoluted_1 = []
        self.cnn_filters_2 = []
        self.cnn_convoluted_2 = []

    def load_images(self):
        self.loaded_images = []
        self.loaded_labels = []

        for i in range(4):
            loaded_images_temp = []
            loaded_labels_temp = []
            for k in range(self.spin_images_n.value()):
                loaded_images_temp.append(1.0 - cv2.cvtColor(cv2.imread(
                    str(self.line_folder.text()) + str(i) + '_' + str(k) + '.bmp'), cv2.COLOR_BGR2GRAY) / 255.0)
                loaded_labels_temp.append([i])
            self.loaded_labels.append(loaded_labels_temp)
            self.loaded_images.append(loaded_images_temp)

        self.loaded_images = np.array(self.loaded_images)
        self.loaded_labels = np.array(self.loaded_labels)

        if self.DEBUG:
            print('-------------------- INPUT DATA --------------------')
            print('Shape of loaded_imagess: ' + str(self.loaded_images.shape))
            print('Shape of loaded_labels: ' + str(self.loaded_labels.shape))
            print('Arrays:')
            print(self.loaded_images)
            print()
            print(self.loaded_labels)
            print('----------------------------------------------------')

    def filter_1_generate(self):
        self.cnn_filters_1 = []
        for i in range(4):
            x_temp = []
            for x in range(5):
                y_temp = []
                for y in range(5):
                    y_temp.append(random.randrange(0, 2))  # {0; 1}
                x_temp.append(y_temp)
            self.cnn_filters_1.append(x_temp)
        self.cnn_filters_1 = np.array(self.cnn_filters_1)
        self.filter_1_show()

    def filter_1_load(self):
        with open(self.line_folder.text() + 'cnn_filters_1.dat', 'rb') as filehandle:
            self.cnn_filters_1 = np.array(pickle.load(filehandle))
        self.filter_1_show()

    def filter_1_save(self):
        with open(self.line_folder.text() + 'cnn_filters_1.dat', 'wb') as filehandle:
            pickle.dump(self.cnn_filters_1, filehandle)

    def filter_1_show(self):
        # filter 1
        image_temp = (self.cnn_filters_1[0]) * 255.0
        image_temp = cv2.resize(image_temp.astype(int), (60, 60), interpolation=cv2.INTER_NEAREST)
        self.cvl_filter_1_1.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(image_temp)))

        # filter 2
        image_temp = (self.cnn_filters_1[1]) * 255.0
        image_temp = cv2.resize(image_temp.astype(int), (60, 60), interpolation=cv2.INTER_NEAREST)
        self.cvl_filter_1_2.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(image_temp)))

        # filter 3
        image_temp = (self.cnn_filters_1[2]) * 255.0
        image_temp = cv2.resize(image_temp.astype(int), (60, 60), interpolation=cv2.INTER_NEAREST)
        self.cvl_filter_1_3.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(image_temp)))

        # filter 4
        image_temp = (self.cnn_filters_1[3]) * 255.0
        image_temp = cv2.resize(image_temp.astype(int), (60, 60), interpolation=cv2.INTER_NEAREST)
        self.cvl_filter_1_4.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(image_temp)))

    def filter_2_generate(self):
        self.cnn_filters_2 = []
        for i in range(4):
            x_temp = []
            for x in range(2):
                y_temp = []
                for y in range(2):
                    y_temp.append(random.randrange(0, 2))  # {0; 1}
                x_temp.append(y_temp)
            self.cnn_filters_2.append(x_temp)
        self.cnn_filters_2 = np.array(self.cnn_filters_2)
        self.filter_2_show()

    def filter_2_load(self):
        with open(self.line_folder.text() + 'cnn_filters_2.dat', 'rb') as filehandle:
            self.cnn_filters_2 = np.array(pickle.load(filehandle))
        self.filter_2_show()

    def filter_2_save(self):
        with open(self.line_folder.text() + 'cnn_filters_2.dat', 'wb') as filehandle:
            pickle.dump(self.cnn_filters_2, filehandle)

    def filter_2_show(self):
        # filter 1
        image_temp = (self.cnn_filters_2[0]) * 255.0
        image_temp = cv2.resize(image_temp.astype(int), (60, 60), interpolation=cv2.INTER_NEAREST)
        self.cvl_filter_2_1.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(image_temp)))

        # filter 2
        image_temp = (self.cnn_filters_2[1]) * 255.0
        image_temp = cv2.resize(image_temp.astype(int), (60, 60), interpolation=cv2.INTER_NEAREST)
        self.cvl_filter_2_2.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(image_temp)))

        # filter 3
        image_temp = (self.cnn_filters_2[2]) * 255.0
        image_temp = cv2.resize(image_temp.astype(int), (60, 60), interpolation=cv2.INTER_NEAREST)
        self.cvl_filter_2_3.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(image_temp)))

        # filter 4
        image_temp = (self.cnn_filters_2[3]) * 255.0
        image_temp = cv2.resize(image_temp.astype(int), (60, 60), interpolation=cv2.INTER_NEAREST)
        self.cvl_filter_2_4.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(image_temp)))

    def apply_filters(self):
        # CONVOLUTION 1
        self.cnn_convoluted_1 = []
        for i in range(4):
            image_array = []
            for k in range(self.spin_images_n.value()):
                single_image = []
                for x in range(16):
                    convoluted_row = []
                    for y in range(16):
                        convoluted_pixel = 0
                        for x_k in range(5):
                            for y_k in range(5):
                                convoluted_pixel += self.loaded_images[i][k][x + x_k][y + y_k] \
                                                    * self.cnn_filters_1[i][x_k][y_k]
                        convoluted_row.append(convoluted_pixel)
                    single_image.append(convoluted_row)
                image_array.append(single_image)
            self.cnn_convoluted_1.append(image_array)

        self.cnn_convoluted_1 = np.array(self.cnn_convoluted_1)
        self.cnn_convoluted_1 /= 25.0

        # Normalization
        for i in range(4):
            for k in range(self.spin_images_n.value()):
                self.cnn_convoluted_1[i][k] = self.cnn_convoluted_1[i][k] - self.cnn_convoluted_1[i][k].min()
                if self.cnn_convoluted_1[i][k].max() > 0:
                    self.cnn_convoluted_1[i][k] = self.cnn_convoluted_1[i][k] *\
                                                  (1.0 / self.cnn_convoluted_1[i][k].max())

        # CONVOLUTION 2
        self.cnn_convoluted_2 = []
        for i in range(4):
            image_array = []
            for k in range(self.spin_images_n.value()):
                single_image = []
                x = 0
                while x <= 14:
                    convoluted_row = []
                    y = 0
                    while y <= 14:
                        convoluted_pixel = 0
                        for x_k in range(2):
                            for y_k in range(2):
                                convoluted_pixel += self.cnn_convoluted_1[i][k][x + x_k][y + y_k] \
                                                    * self.cnn_filters_2[i][x_k][y_k]
                        convoluted_row.append(convoluted_pixel)
                        y += 2
                    single_image.append(convoluted_row)
                    x += 2
                image_array.append(single_image)
            self.cnn_convoluted_2.append(image_array)

        self.cnn_convoluted_2 = np.array(self.cnn_convoluted_2)
        self.cnn_convoluted_2 /= 4.0

        # Normalization
        for i in range(4):
            for k in range(self.spin_images_n.value()):
                self.cnn_convoluted_2[i][k] = self.cnn_convoluted_2[i][k] - self.cnn_convoluted_2[i][k].min()
                if self.cnn_convoluted_2[i][k].max() > 0:
                    self.cnn_convoluted_2[i][k] = self.cnn_convoluted_2[i][k] * \
                                                  (1.0 / self.cnn_convoluted_2[i][k].max())
        self.preview_filters()

    def filters_load(self):
        # noinspection PyBroadException
        try:
            print('Loading filters and convolutions from file...')
            self.filter_1_load()
            self.filter_2_load()
            with open(self.line_folder.text() + 'cnn_convolution_1.dat', 'rb') as filehandle:
                self.cnn_convoluted_1 = np.array(pickle.load(filehandle))
            with open(self.line_folder.text() + 'cnn_convolution_2.dat', 'rb') as filehandle:
                self.cnn_convoluted_2 = np.array(pickle.load(filehandle))
            self.preview_filters()
            print('Done.')
        except:
            print(sys.exc_info())

    def filters_save(self):
        print('Saving filters and convolutions to file...')
        self.filter_1_save()
        self.filter_2_save()
        with open(self.line_folder.text() + 'cnn_convolution_1.dat', 'wb') as filehandle:
            pickle.dump(self.cnn_convoluted_1, filehandle)
        with open(self.line_folder.text() + 'cnn_convolution_2.dat', 'wb') as filehandle:
            pickle.dump(self.cnn_convoluted_2, filehandle)
        print('Done.')

    def preview_filters(self):
        src_data = self.loaded_images[self.spin_preview_from.value()][self.spin_preview_n.value()]
        filter_data_1 = self.cnn_filters_1[self.spin_preview_from.value()]
        result_data_1 = self.cnn_convoluted_1[self.spin_preview_from.value()][self.spin_preview_n.value()]
        filter_data_2 = self.cnn_filters_2[self.spin_preview_from.value()]
        result_data_2 = self.cnn_convoluted_2[self.spin_preview_from.value()][self.spin_preview_n.value()]

        src_image = (src_data * 255.0).astype(int)
        filter_image_1 = (filter_data_1 * 255.0).astype(int)
        result_image_1 = (result_data_1 * 255.0).astype(int)
        filter_image_2 = (filter_data_2 * 255.0).astype(int)
        result_image_2 = (result_data_2 * 255.0).astype(int)

        self.ocl_preview_src.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
            cv2.resize(src_image, (120, 120), interpolation=cv2.INTER_NEAREST))))
        self.ocl_preview_filter.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
            cv2.resize(filter_image_1, (30, 30), interpolation=cv2.INTER_NEAREST))))
        self.ocl_preview_result.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
            cv2.resize(result_image_1, (96, 96), interpolation=cv2.INTER_NEAREST))))
        self.ocl_preview_src_2.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
            cv2.resize(result_image_1, (96, 96), interpolation=cv2.INTER_NEAREST))))
        self.ocl_preview_filter_2.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
            cv2.resize(filter_image_2, (12, 12), interpolation=cv2.INTER_NEAREST))))
        self.ocl_preview_result_2.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
            cv2.resize(result_image_2, (48, 48), interpolation=cv2.INTER_NEAREST))))

    def predict_test_image(self):
        # noinspection PyBroadException
        try:
            # self.load_images()
            # self.filters_load()
            # self.load_model_from_file()

            random_index = random.randrange(0, self.spin_images_n.value())
            temp_data = self.cnn_convoluted_2[self.spin_test_array_id.value()][random_index]

            temp_image = (temp_data * 255.0).astype(int)
            self.ocl_test_image.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
                cv2.resize(temp_image, (128, 128), interpolation=cv2.INTER_NEAREST))))

            test_array = np.array([temp_data.flatten()])

            test_labels = np.array([[self.spin_test_array_id.value()]])
            output_l0 = test_array
            output_l1 = dot_0_layer(output_l0, self.synaptic_weights_0)
            output_l2 = sigmoid(np.dot(output_l1, self.synaptic_weights_1.T))
            if np.argmax(output_l2[0]) == test_labels[0][0]:
                self.label_predicted.setText(str(int(np.argmax(output_l2[0])) + 1) + ' YEAH')
            else:
                self.label_predicted.setText(str(int(np.argmax(output_l2[0])) + 1) + ' NOPE')
            self.progressBar_2.setValue(output_l2[0][0] * 100)
            self.progressBar_3.setValue(output_l2[0][1] * 100)
            self.progressBar_4.setValue(output_l2[0][2] * 100)
            self.progressBar_5.setValue(output_l2[0][3] * 100)

            # Preview CNN
            cnn_preview_image = 255 * np.ones((512, 512, 3), dtype=np.uint8)
            y = int((512 - 256) / 2)
            for i in range(64):
                color = int((1 - test_array[0][i]) * 255)
                cv2.circle(cnn_preview_image, (10, y + 4 * i), 2, (color, color, color), -1)

            y = int((512 - 256) / 2)
            for i in range(512):
                for k in range(64):
                    calculated = self.synaptic_weights_0[i][k] * output_l0[0][k]
                    below_zero = False if calculated > 0 else True
                    saturation = calculated * 255 if not below_zero else 255
                    value = calculated * -255 if below_zero else 255

                    color_hsv = np.uint8([[[k * 2, saturation, value]]])
                    color_rgb = cv2.cvtColor(color_hsv, cv2.COLOR_HSV2RGB)[0][0]
                    if calculated != 0:
                        cv2.line(cnn_preview_image, (254, i), (11, y + 4 * k),
                                 (int(color_rgb[0]), int(color_rgb[1]), int(color_rgb[2])))

            y = int((512 - 80) / 2)
            for i in range(512):
                for k in range(4):
                    calculated = self.synaptic_weights_1[k][i] * output_l1[0][i]
                    below_zero = False if calculated > 0 else True
                    saturation = calculated * 127 if not below_zero else 255
                    value = calculated * -127 if below_zero else 255

                    color_hsv = np.uint8([[[k * 40, saturation, value]]])
                    color_rgb = cv2.cvtColor(color_hsv, cv2.COLOR_HSV2RGB)[0][0]
                    if calculated != 0:
                        cv2.line(cnn_preview_image, (256, i), (500, y + 20 * k),
                                 (int(color_rgb[0]), int(color_rgb[1]), int(color_rgb[2])))

            y = int((512 - 80) / 2)
            for i in range(4):
                saturation = output_l2[0][i] * 255
                color_hsv = np.uint8([[[i * 40, saturation, 255]]])
                color_rgb = cv2.cvtColor(color_hsv, cv2.COLOR_HSV2RGB)[0][0]

                cv2.circle(cnn_preview_image, (500, y + 20 * i + 1), 5,
                           (int(color_rgb[0]), int(color_rgb[1]), int(color_rgb[2])), -1)

            for i in range(512):
                calculated = output_l1[0][i]
                below_zero = False if calculated > 0 else True
                saturation = calculated * 255 if not below_zero else 255
                value = calculated * -255 if below_zero else 255
                color_hsv = np.uint8([[[255, saturation, value]]])
                color_rgb = cv2.cvtColor(color_hsv, cv2.COLOR_HSV2RGB)[0][0]
                if calculated != 0:
                    cv2.circle(cnn_preview_image, (255, i), 4,
                               (int(color_rgb[0]), int(color_rgb[1]), int(color_rgb[2])), -1)

            self.ocl_preview_cnn.setPixmap(QPixmap.fromImage(qimage2ndarray.array2qimage(
                cv2.resize(cnn_preview_image, (512, 512), interpolation=cv2.INTER_NEAREST))))

        except:
            print(sys.exc_info())

    def save_model_to_file(self):
        compressed_data = [self.synaptic_weights_0, self.synaptic_weights_1]
        with open(self.line_folder.text() + 'model.dat', 'wb') as filehandle:
            pickle.dump(compressed_data, filehandle)

    def load_model_from_file(self):
        with open(self.line_folder.text() + 'model.dat', 'rb') as filehandle:
            compressed_data = pickle.load(filehandle)
            self.synaptic_weights_0 = np.array(compressed_data[0])
            self.synaptic_weights_1 = np.array(compressed_data[1])

        print('-------------------- WEIGHTS --------------------')
        print('Shape of synaptic_weights_0: ' + str(self.synaptic_weights_0.shape))
        print('Shape of synaptic_weights_1: ' + str(self.synaptic_weights_1.shape))
        print('Arrays:')
        print(self.synaptic_weights_0)
        print()
        print(self.synaptic_weights_1)
        print('-------------------------------------------------')

    def start_training(self):
        if len(self.synaptic_weights_0) == 0 or len(self.synaptic_weights_1) == 0:
            # Synaptic weights arrays
            self.synaptic_weights_0 = []
            for i in range(512):  # 4096
                string_array = [int(random.randrange(-1, 2)) for _ in range(3)] + [0 for _ in range(61)]
                random.shuffle(string_array)
                self.synaptic_weights_0.append(string_array)
            self.synaptic_weights_0 = np.array(self.synaptic_weights_0)
            self.synaptic_weights_1 = np.array(2 * np.random.random((4, 512)) - 1)  # 4096
            if self.DEBUG:
                print('-------------------- WEIGHTS --------------------')
                print('Shape of synaptic_weights_0: ' + str(self.synaptic_weights_0.shape))
                print('Shape of synaptic_weights_1: ' + str(self.synaptic_weights_1.shape))
                print('Arrays:')
                print(self.synaptic_weights_0)
                print()
                print(self.synaptic_weights_1)
                print('-------------------------------------------------')

        thread = threading.Thread(target=self.training)
        thread.start()

    def training(self):
        self.cnn_convoluted_2 = np.array(self.cnn_convoluted_2)
        self.loaded_labels = np.array(self.loaded_labels)
        train_data = []
        train_labels = []
        for i in range(4):
            for k in range(self.spin_images_n.value()):
                train_data.append(self.cnn_convoluted_2[i][k].flatten())
        for i in range(4):
            for k in range(self.spin_images_n.value()):
                train_labels.append(self.loaded_labels[i][k][0])
        train_data = np.array(train_data)
        train_labels = np.array([train_labels])

        if self.DEBUG:
            print('-------------------- TRAIN DATA --------------------')
            print('Shape of train_data: ' + str(train_data.shape))
            print('Shape of train_labels: ' + str(train_labels.shape))
            print('Arrays:')
            print(train_data)
            print()
            print(train_labels)
            print('----------------------------------------------------')

        # noinspection PyBroadException
        try:
            i = 0
            while i < int(self.spin_iterations.value()):
                output_l0 = train_data
                output_l1 = dot_0_layer(output_l0, self.synaptic_weights_0)
                output_l2 = sigmoid(np.dot(output_l1, self.synaptic_weights_1.T))

                # Layer 2 error calculations
                error_l2 = []
                for k in range(len(output_l2)):
                    a = []
                    for m in range(4):
                        if m == train_labels[0][k]:
                            a.append(1 - output_l2[k][m])
                        else:
                            a.append(0 - output_l2[k][m])
                    error_l2.append(a)
                error_l2 = np.array(error_l2)

                adjustments_l2 = output_l1.T.dot(error_l2 * (output_l2 * (1 - output_l2)))
                self.synaptic_weights_1 += adjustments_l2.T

                # Accuracy calculations
                predicted = []
                accuracy = 0
                for k in range(len(output_l2)):
                    predicted.append(np.argmax(output_l2[k]))
                    if np.argmax(output_l2[k]) == train_labels[0][k]:
                        accuracy += 1
                accuracy /= len(output_l2)
                predicted = np.array(predicted)

                if i % 1 == 0:
                    print('-------------------- I: ' + str(i) + ' --------------------')
                    # print('output_l2: ' + str(output_l2))
                    # print('error_l2: ' + str(error_l2))
                    print('predicted: ' + str(predicted))
                    print('accuracy: ' + str(accuracy))
                    # print('adjustments_l2: ' + str(adjustments_l2))
                    # print('----------------------------------------------')
                i += 1
                self.progressBar.setValue(valmap(i, 0, self.spin_iterations.value(), 0, 100))

            self.progressBar.setValue(0)

        except:
            print(sys.exc_info())


if __name__ == '__main__':
    app = QtWidgets.QApplication(sys.argv)
    app.setStyle("Fusion")

    window = CNNApp()
    window.show()
    app.exec_()